XMotos Mini GP Race Bike

Less than 100miles of use. Like new Condition.

Includes:

Extra Set of Body Plastic

Bike Stand

This Bike was imported parts available online from XMotos.

Adjustable forks

125cc GP shift motor

Adjustable shock

1/4 turn throttle

Adjustable rear sets

Electric start

Front and rear disk brakes

Fiberglass body

10" wheels

Mukine Carb

OTHER SPECS:

Engine: Single cylinder, 4-stroke, air-cooling

Bore Stroke: 47mm-49.5mm

Compression Ratio: 8.5:1

Ignition :CDI

Rated Power: 3.7/8000(±500)

Max Power: 3.7/8000(±500)

Max Torque: 5.5/5000(±500)

Clutch Method: Manual Wet Multi-Plate

Lubrication: Force-feed, splash

Start Method: Electric/kick

DIMENSIONS

L - W - H: 1045--560--910mm

Wheel Base: 1085 mm

Seat Height: 650 mm

Ground Clearance: 200 mm

BODYWORK

Frame Steel

Front Suspension: length 640mm, travel 100mm

Rear Suspension: Single shock, adjustable, 250 lbs.

Chain: 420/94#

Tire: Front 90/90/10; Rear 3.5/10

brake : Disc, Frontφ190 mm, Rearφ180 mm

Tank Capacity: 3L

Rated Load: 120kg

Net Weight: 75±1kg